CPE301 – SPRING 2019

# Design Assignment X

Student Name: Moriah Wingrove

Student #: 5000156247

Student Email: Wingrove@unlv.nevada.edu

Primary Github address: https://github.com/windew/Tiny_Dragons.git

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

No components were used for design assignment 2A due to not having the microcontroller and necessary parts from the mail or put together.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2/A TASK 1

Asm code

;

; design_assignment_2A.asm

;

; Created: 9/27/2019 2:14:43 PM

; Author :Moriah Wingrove

; Replace with your application code

.include<m328pbdef.inc>

;Task One

;Design delay subroutine to generate waveform on portb.3 with 40% duty cycle and 0.625 sec per period

sbi DDRB, 3           ; set Port b to output

blink:

sbi portb, 3          ; turn on LED on Portb bit 3

rcall delay_250ms     ; delay will be 250ms

cbi portb, 3          ; turn off LED on  portb bit 3

rcall delay_375ms      ; delay will be 375 ms

rjmp blink            ; loop back to the start

; Delay 4 000 000 cycles

; 250ms at 16 MHz

delay_250ms:

ldi  r18, 21	;r18 = 21

ldi  r19, 75	;r19 = 75

ldi  r20, 191	;r20 = 191

L1: dec  r20	;decremenet r20

brne L1		;if r20 != 0 go again

dec  r19	;decrement r19

brne L1		;if r19 != 0 go again

dec  r18	;decrement r18

brne L1		;if r18 != 0 go again

nop

ret

; Delay 6 000 000 cycles

; 375ms at 16 MHz

delay_375ms:

ldi  r18, 31	;r18 = 31

ldi  r19, 113	;r19 = 113

ldi  r20, 31	;r20 = 31

L2: dec  r20		;decremenet r20

brne L2		;if r20 != 0 go again

dec  r19		;decrement r19

brne L2			;if r19 != 0 go again

dec  r18		;decrement r18

brne L2		;if r18 != 0 go again

nop

ret

C code

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRB |= 0b1000;        // set PORTB.3 for output

while (1)

{

_delay_ms(250);       // delay for 250ms

PORTB |= 0b1000;    // set LED on

_delay_ms(375);      // delay for remaining 375ms

PORTB &= ~0b1000;    // set LED off

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 2

Asm code

;

; Design_Assignment_2A_task2.asm

;

; Created: 9/30/2019 9:20:31 PM

; Author : Moriah Wingrove

;

; Replace with your application code

;Task Two

;Connect a switch to portc.3(active high -turn on and pull up transistor) to poll for and event to turn on led at portb.2 for 1.33 sec after the event

.org 0

cbi DDRC, 3 ;sets port c as input

sbi portc, 3 ; sets port c bit 2 high to enable pull up

sbi ddrb, 2 ;sets port b as output

cbi portb, 2  ;LED is off

poll_loop:

sbic pinc, 3	;if pinc is low (0) skip next instruction

jmp LED			;jumps to label led

sbi portb, 2	;turn on led

; 1s 333ms at 16 MHz

ldi  r18, 109		;r18 = 109

ldi  r19, 51		;r19 = 51

ldi  r20, 106		;r20 = 106

L1: dec  r20			;decrement r20

brne L1				;if r20 != 0 go to label l1

dec  r19			;decrement r19

brne L1				;if r20 != 0 go to label l1

dec  r18			;decrement r19

brne L1				;if r18 != 0 go to label l1

LED:

cbi portb, 2	 ;turns led off

jmp poll_loop	;go to label poll_loop

C code

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRC &= 0b1000;            // connect PORTC.2 to switch as input

PORTC |= 0b1000;     // enable pull-up on port c

DDRB |= 0b0100;            // set PORTB.2 for output

while(1){

if(!(PINC & (1<<PINC2))){  // check if pin is low

PORTB &= ~0b0100;          // set LED on

_delay_ms(1333);           // delay of 1.33 sec

}

else{                                    // otherwise (pin is high)

PORTB |= 0b0100;           // LED off

}

}

return 0;

}

- SCHEMATICS

No schematics due to lack of microcontroller

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

TASK ONE:

Figure 1: before portb.3 is set watch is at 0.13us

Figure 2: DDRB set to output

Figure 3: Pinb and portb.3 are set meaning the LED has been turned on

Figure 4: The time on the stopwatch is 250,000us or 250 ms

Figure 5: The led has been turned off and one period has been completed. The time on the watch is 625,000us or 625 ms

TASK TWO:

Figure 6: DDRC is set as an input and portc is set high

Figure 7: DDRB is set as an output and portb is cleared

Figure 8: Initial stopwatch before the led is turned on time is 0.69us

Figure 9: the poll event has occurred and the led is turned on portb.2 is set

Figure 10: The time in which the led is on is 1,333,000.81 us or 1.333s

Figure 11: LED has been turned off portb2 is cleared

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Board was not set up due to lack of microcontroller

- VIDEO LINKS OF EACH DEMO

There are no videos due to the lack of working microcontroller

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT