CPE301 – SPRING 2019

# Design Assignment X

Student Name:

Student #:

Student Email:

Primary Github address:

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2/A

; Design_Assignment_2A_task2.asm

;

; Created: 9/30/2019 9:20:31 PM

; Author : Moriah Wingrove

;

; Replace with your application code

;Task Two

;Connect a switch to portc.3(active high -turn on and pull up transistor) to poll for and event to turn on led at portb.2 for 1.33 sec after the event

.org 0

cbi DDRC, 3 ;sets port c as input

sbi portc, 3 ; sets port c bit 2 high to enable pull up

sbi ddrb, 2 ;sets port b as output

cbi portb, 2  ;LED is off

poll_loop:

sbic pinc, 3	;if pinc is low (0) skip next instruction

jmp LED			;jumps to label led

sbi portb, 2	;turn on led

; 1s 333ms at 16 MHz

ldi  r18, 109		;r18 = 109

ldi  r19, 51		;r19 = 51

ldi  r20, 106		;r20 = 106

L1: dec  r20			;decrement r20

brne L1				;if r20 != 0 go to label l1

dec  r19			;decrement r19

brne L1				;if r20 != 0 go to label l1

dec  r18			;decrement r19

brne L1				;if r18 != 0 go to label l1

LED:

cbi portb, 2	 ;turns led off

jmp poll_loop	;go to label poll_loop

- DEVELOPED MODIFIED CODE OF TASK 2/B from TASK 2/A

;

; Design_Assignment_2B.asm

;

; Created: 10/5/2019 10:02:00 AM

; Author : Moriah Wingrove

;

; TASK ONE: Implement design assignment 2A.2 using INTO (PD2 pin) interrupt mechanism

;			2A.2: connect switch to PORTC.3(active high - turn on to pull up transistor) to poll for even to turn on

;			      LED at PORTB.2 for 1.333 seconds after event

.include <m328pbdef.inc>	;include file for atmega328PB

.org 0x00	;location for reset

jmp main	;begin main program

.org 0X02	;location of interupt PD2

jmp EXO_ISR	;external interrupt program

main:

ldi r20, high(RAMEND)		;initialize stack

out sph, r20				;initialize stack

ldi r20, low(RAMEND)		;initialize stack

out spl, r20				;initialize stack

ldi r20, 0x2	;make INTO falling edge triggered

sts EICRA, r20

sbi portd, 2	;pull up activated

cbi DDRC, 3		;sets port c as input for switch

sbi portc, 3	; sets port c bit 2 low switch not connected

sbi ddrb, 2		;sets port b as output

cbi portb, 2	;LED is off

ldi r20, 1<<INT0 ;enable INT0

out EIMSK, r20

SEI			;enable interrupts

wait:

jmp wait	;wait for interrupt

EXO_ISR:

in r21, portc

sbic pinc, 3	;if pinc is low (0) skip next instruction

jmp LED			;jumps to label led

sbi portb, 2	;turn on led

out portc, r21

; 1s 333ms at 16 MHz

ldi  r18, 109		;r18 = 109

ldi  r19, 51		;r19 = 51

ldi  r20, 106		;r20 = 106

L1: dec  r20			;decrement r20

brne L1				;if r20 != 0 go to label l1

dec  r19			;decrement r19

brne L1				;if r20 != 0 go to label l1

dec  r18			;decrement r19

brne L1				;if r18 != 0 go to label l1

LED:	cbi portb, 2	 ;turns led off

RETI

C CODE

/*

* Design_Assignment_2B_C.c

*

* Created: 10/6/2019 12:00:49 AM

* Author : Moriah

*/

#include <avr/io.h>

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRC &= 0b1000;            // connect PORTC.2 to switch as input

PORTC |= 0b1000;     // enable pull-up on port c

DDRB |= 0b0100;            // set PORTB.2 for output

PORTD = 1<<2;		// pull up activated

EICRA = 0x2;	//make INT0 falling edge

EIMSK = (1<<INT0);	//enable external interrupt 0

sei (); //enable interrupts

while(1); //wait here for interrupt

}

ISR (INT0_vect)	//isr for external interrup 0

if(!(PINC & (1<<PINC2))){  // check if pin is low

PORTB &= ~0b0100;          // set LED on

_delay_ms(1333);           // delay of 1.33 sec

}

else{                                    // otherwise (pin is high)

PORTB |= 0b0100;           // LED off

return

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Figure 1: debugging code

Figure 2: portb2 is set as output

Figure 3: portc is an input portc.3 active high

Figure 4: led on portb.2 is on time is 1.81us

Figure 5: LED has been turned of 1.333 seconds after interrupt

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA