CPE301 – SPRING 2019

# Design Assignment 2C

Student Name: Moriah Wingrove

Student #: 5000156247

Student Email: Wingrove@unlv.nevada.edu

Primary Github address: https://github.com/windew/Tiny_Dragons.git

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

No components were used for design assignment 2C due to not having the microcontroller and necessary parts from the mail or put together.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2/C TASK1 2A.1

Asm code

;

; Design_Assignment_2C_asm.asm

;

; Created: 10/10/2019 9:56:13 PM

; Author : Moriah Wingrove

;

; Goal: Use GPIO and delays using timers and interupts:

;TASK 1: Implement design assignment 2A using Timer 0 - normal mode. Count OVF occurrence if needed. Do not use interrupts

;2A was to generate a waveform on portb.3 with 40% DC and 0.625s period

begin:

sbi ddrb,3	;pb3 is an output

cbi portb, 3	;Led is off

ldi r23, 0x61

timer:

ldi r20, 0x0f	;r20 = 0

out tcnt0, r20  ;load timer 0

ldi r22, 0x00

out tccr0a, r22	; timer0, normal

ldi r21, (1<<CS02)  ;

out TCCR0B, r21;   send r21 out to timer

check:

in r21, TIFR0	;read overflow

sbrs r21, 0;   skip if overflow occurs

jmp check

out tccr0b, r22 ; stop timer

ldi r21, (1<<tov0)

out TIFR0, r21	;clear overflow

dec r23

brne timer

blink:

ldi r23,0x41

sbi portb, 3          ; turn on LED on Portb bit 3

timer2:

ldi r20, 0x0f	;r20 = 0

out tcnt0, r20  ;load timer 0

ldi r22, 0x00

out tccr0a, r22	; timer0, normal

ldi r21, (1<<CS02)  ; prescalar 256

out TCCR0B, r21;   send r21 out to timer

check2:

in r21, TIFR0	;read overflow

sbrs r21, 0;   skip if overflow occurs

jmp check2

out tccr0b, r22 ; stop timer

ldi r21, (1<<tov0)

out TIFR0, r21	;clear overflow

dec r23

brne timer2

rjmp begin            ; loop back to the start

C code

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

/*

* DA2C_1_C.c

*

* Created: 10/13/2019 12:42:27 AM

* Author : Moriah Wingrove

*/

#include <avr/io.h>

int main(void)

{

unsigned char count = 0;  //count to keep track of timer0

DDRB |= (1<<3);    //portb.3 output of LED

PORTB = 0;		// led is off

while (1)

{

//set timer0 with prescalar 256 and normal mode

TCCR0A = 0;

TCCR0B |= (1<<CS02);

TCNT0 = 0;		//counter initiallized

while((TIFR0 & 0X1) == 0);  //WAIT for overflow

TCCR0B = 0; //stop timer

TIFR0 |= 1; //clear overflow

if (count > 90)   //edit

{

PORTB |= (1<<3); //led is on

if(count < 152 )

{

count++;

}

else

{

count = 0;

PORTB = 0; //led is off

count++;

}

}

else

count++;

}

}

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2/C TASK1 2A.2

;

; Design_Assignment_2C_T1_2A2_asm.asm

;

; Created: 10/14/2019 12:14:14 AM

; Author : Moriah Wingrove

;

;TASK 1: Implement design assignment 2A using Timer 0 - normal mode. Count OVF occurrence if needed. Do not use interrupts

;2A Connect a switch to PORTC.3 (active high - turn on the pull up transistor)

;to poll for an event to turn on the led at PORTB.2 for 1.333 sec after the event.

begin:

cbi DDRC, 3 ;sets port c as input

sbi portc, 3 ; sets port c bit 2 high to enable pull up

sbi ddrb, 2 ;sets port b as output

cbi portb, 2  ;LED is off

poll_loop:

sbic pinc, 3	;if pinc is low (0) skip next instruction

jmp LED			;jumps to label led

sbi portb, 2	;turn on led

ldi r23, 0x5a

ldi r24, 0x02

timer:

ldi r20, 0x0f	;r20 = 0

out tcnt0, r20  ;load timer 0

ldi r22, 0x00

out tccr0a, r22	; timer0, normal

ldi r21, (1<<CS02)  ;

out TCCR0B, r21;   send r21 out to timer

check:

in r21, TIFR0	;read overflow

sbrs r21, 0;   skip if overflow occurs

jmp check

out tccr0b, r22 ; stop timer

ldi r21, (1<<tov0)

out TIFR0, r21	;clear overflow

dec r23

brne timer

dec r24

brne timer

LED:

cbi portb, 2	 ;turns led off

jmp poll_loop	;go to label poll_loop

C code

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

/*

* DA2C_1_C.c

*

* Created: 10/13/2019 12:42:27 AM

* Author : Moriah Wingrove

*/

#include <avr/io.h>

int main(void)

{

unsigned char count = 0;  //count to keep track of timer0

DDRB |= (1<<3);    //portb.3 output of LED

PORTB = 0;		// led is off

while (1)

{

//set timer0 with prescalar 256 and normal mode

TCCR0A = 0;

TCCR0B |= (1<<CS02);

TCNT0 = 0;		//counter initiallized

while((TIFR0 & 0X1) == 0);  //WAIT for overflow

TCCR0B = 0; //stop timer

TIFR0 |= 1; //clear overflow

if (count > 90)   //edit

{

PORTB |= (1<<3); //led is on

if(count < 152 )

{

count++;

}

else

{

count = 0;

PORTB = 0; //led is off

count++;

}

}

else

count++;

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/C from TASK 2

Asm code

;

; Design_Assignment_2C_T2_asm.asm

;

; Created: 10/13/2019 1:37:35 AM

; Author : Moriah

;

;TASK 2: Implement design assignment 2A using Timer0_OVF_vect interrupt mechanism in normal mode.

;2A was to generate a waveform on portb.3 with 40% DC and 0.625s period

.org 0				;location of main prgm

jmp main		;go to main prgm

.org 0x20			;location on interrupt

jmp T0_OVF		;jmp to interrupt

main:

sbi ddrb,3		;pb3 is an output

cbi portb, 3	;Led is off

ldi r23, 0x58

timer:

ldi r20, 0x0	;r20 = 0

out tcnt0, r20  ;load timer 0

ldi r22, 0x00

out tccr0a, r22	; timer0, normal

ldi r21, (1<<CS02) |(1<<CS00)  ;

out TCCR0B, r21;   send r21 out to timer

;enable interrupts

ldi r20, 0x01	;r20 =1

sts TIMSK0, r20; store r20 in TIMSKO - enables interrupt overflow

sei          ; interrupts enabled

wait:

jmp wait   ;loop until interrupt occurs

T0_OVF:

out tccr0b, r22 ; stop timer

ldi r21, (1<<tov0)

out TIFR0, r21	;clear overflow

dec r23			;timer needs to overflow

brne done

sbi portb, 3	;turn on led

ldi r23, 0x3e

timeron:

ldi r20, 0x00	;r20 = 0

out tcnt0, r20  ;load timer 0

ldi r21, (1<<CS02)  ;

out TCCR0B, r21;   send r21 out to timer

check:

in r21, TIFR0	;read overflow

sbrs r21, 0;   skip if overflow occurs

jmp check

out tccr0b, r22 ; stop timer

ldi r21, (1<<tov0)

out TIFR0, r21	;clear overflow

dec r23

brne timeron

rjmp main

done:

ldi r20, 0x00	;r20 = 0

out tcnt0, r20  ;load timer 0

ldi r21, (1<<CS02)  ;

out TCCR0B, r21;   send r21 out to timer

reti

C code

/*

* design_assignment_2C_T2_c.c

*

* Created: 10/13/2019 1:48:29 PM

* Author : Moriah Wingrove

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/io.h>

#include <avr/interrupt.h>

// 1Mhz clock & 64 prescaler

// global variable for keeping track of # of times Timer0 overflows

volatile int count;

// this interrupt service routine (ISR) runs whenever an overflow on Timer0 occurs

int main(void) {

count = 0;  // initialize count to keep track of number of

DDRB |= (1 << 3);   // connect LED to pin PB.2

PORTB = 0; //led is off

// set up Timer0 with prescaler = 1024 and normal mode

TCCR0A = 0;

TCCR0B |= (1 << CS02)|(1 << CS00);

TCNT0 = 0;    // initialize counter

TIMSK0 |= (1 << TOIE0);  // enable overflow interrupt

sei();     // enable global interrupts

while(1) ;    // loop forever

}

ISR (TIMER0_OVF_vect)

{

if (count > 21)

{

PORTB |= (1 << 3);   // Toggle PB.3

if(count < 37 )

{

count++;

}

else

{

count = 0;

PORTB = 0;

count++;  // reinitialize count

}

}

else  count++;

}

- DEVELOPED MODIFIED CODE OF TASK 2/C from TASK 3

Asm code

;

; Design_Assignment_2C_T3_asm.asm

;

; Created: 10/13/2019 3:29:25 AM

; Author : Moriah Wingrove

;

.org 0x00 ;reset location

jmp main

.org 0x1c	;location for timer0 cmpA

jmp t0_cm_isr

main:

ldi r23, 23 ;MAKE 23 FOR 375 DELAY

ldi r24,0x3e

cbi portb,3  ;led off

timer:

OUT	SPH,R20

LDI	R20,LOW(RAMEND)

OUT	SPL,R20

LDI	R20,0xff

OUT	OCR0A,R20	;OCR0A

LDI	R20,0x02	;(1<<WGM01)

OUT	OCR0B,R20	;Timer in CTC mode

; Force Output Compare A

LDI	R20,0x0d	 ;

OUT	TCCR0B,R20     ; Force Output & Start Timer0

SBI	DDRB,3	     ;PB3 as an output

LDI	R20,(1<<OCIE0A) ;Timer0 compare match

sts	TIMSK0, r20

SEI	;Set I – SET Main Interrupt Enable after all config

wait:	JMP	wait

;--------ISR for Timer 0

t0_cm_isr:

LDI R20, (1<<OCF0A)

OUT	TIFR0, R20

dec r23			;timer needs to overflow

brne timer

sbi portb,3

timeron:

ldi r21, (1<<tov0)

out TIFR0, r21	;clear overflow

ldi r20, 0x00	;r20 = 0

out tcnt0, r20  ;load timer 0

ldi r21, (1<<CS02)  ;

out TCCR0B, r21;   send r21 out to timer

check:

in r21, TIFR0	;read overflow

sbrs r21, 0;   skip if overflow occurs

jmp check

out tccr0b, r22 ; stop timer

ldi r21, (1<<tov0)

out TIFR0, r21	;clear overflow

dec r24

brne timeron

rjmp main

- SCHEMATICS

No schematics due to lack of microcontroller

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

TASK 2C_T1_2A.1:

Figure 1: before portb.3 is set watch is at 0.13us

Figure 2: led is off for 374ms

Figure 3: portb3 is set led is on

Figure 4: The led has been turned on for 250ms and total period is 625ms

TASK 2C_T1_2A.2:

Figure 5:Before code is debugged

Figure 6: led on portb.2 is on

Figure 7: led turns off after 1.333s

TASK 2C_T2_2A.1:

Figure 8: DDRB is set as output – LED is off

Figure 9: after delay of 372ms the led turns on

Figure 10: stopwatch is 626ms and led turns off

TASK 3.1:

Figure 11: ddrb is output

Figure 12: after 376ms LED turns on

Figure 13: after 250ms the led turns off - total period is 630ms

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Board was not set up due to lack of microcontroller

- VIDEO LINKS OF EACH DEMO

There are no videos due to the lack of working microcontroller

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT