CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Moriah Wingrove

Student #: 5000156247

Student Email: Wingrove@unlv.nevada.edu

Primary Github address: https://github.com/windew/Tiny_Dragons.git

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega 328pb

FTDI 232 chip

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2/C TASK1 2A.1

C code

/*

* Design_Assignment_3A.c

*

* Created: 10/13/2019 4:11:17 AM

* Author : Moriah Wingrove

*/

//Write a C AVR program that will display a string, random integer and floating point values on the serial terminal

// every 1 sec. Use a timer with interrupt for the 1 sec delay. Use a FTDI chip for serial to USB conversion

#define BAUD 9600			//Defines baud rate

#define F_CPU 16000000UL	//Defines clock of 16MHz

#include <avr/io.h>

#include <stdio.h>

#include <util/delay.h>

#include <avr/interrupt.h>

void USART_send(char data);	// Used to send integer to terminal

void USART_putstring(char* StringPtr); // Used to take in every character in the string and sends it to the terminal

void USART_init(void); // Initializes the analog to digital functions, as well as OVF interrupt

char stringtype[] = "String: "; // Declaring the string value on screen

char inttype[] = "Integer: "; // Declaring the integer value on screen

char floattype[] = "Floating Point: "; // Declaring the floating point value on screen

char Space[] = "\n"; // Used to create the next line

char String[] = "String for assignment. Working."; //String[] is the variable to output into terminal

char outs[20]; // Allocating memory space to contain the float value

volatile float adc_temp = 95; // Sets the float value

int main(void)

{

USART_init(); // Initializes the analog to digital functions as well as OVF interrupt

while(1)

{

// main loop

}

}

ISR (TIMER1_OVF_vect)

{

USART_putstring(Space); // creates next line

USART_putstring(stringtype); // LABEL PRINT "String: "

USART_putstring(String); // prints string "String for assignment. Working."

USART_putstring(Space); // creates next line

USART_putstring(inttype); // LABEL PRINT "Integer: "

USART_send('3'); // prints value 3

USART_putstring(Space); // creates next line

USART_putstring(floattype); // LABEL PRINT "floating point: "

snprintf(outs, sizeof(outs), "%f\r\n", adc_temp); // the floating point characters are stored in outs

USART_putstring(outs); // transmits outs to UART

USART_putstring(Space); // creates next line

TCNT1 = 49911; // Reset timer

}

void USART_init( void )

{

UBRR0H = 0;

UBRR0L = F_CPU/16/BAUD - 1; // Used for the BAUD prescaler

UCSR0C = _BV(UCSZ01) | _BV(UCSZ00); /* 8-bit data */

UCSR0B = _BV(RXEN0) | _BV(TXEN0);   /* Enable RX and TX */

TCCR1B |= 5; //(1 << CS12) | (1 << CS10); // Sets prescaler to 1024

TIMSK1 = (1 << TOIE1); // Enables overflow flag

TCNT1 = 49911; // 1 second delay = (0xFFFF) - TCNT = 65535 - 15624 = 49911

sei();

}

void USART_send(char data)

{

while (!(UCSR0A & (1 << UDRE0))); // Until UDRE0 goes high, it will keep looping

UDR0 = data; // UDR0 register grabs the value given from the parameter

}

void USART_putstring(char *StringPtr)

{

while ((*StringPtr != '\0')){ // Until it reaches the end of the line, it will keep looping

while (!(UCSR0A & (1 << UDRE0))); // Until UDRE0 goes high, it will keep looping

UDR0 = *StringPtr; // UDR0 register grabs the value given from the parameter

StringPtr++; // but it does it by every character as shown here

}

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

https://github.com/windew/Tiny_Dragons

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT