CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Moriah Wingrove

Student #: 5000156247

Student Email: Wingrove@unlv.nevada.edu

Primary Github address: https://github.com/windew/Tiny_Dragons.git

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega 328pb

FTDI 232 chip

HiLetgo Multi-Function Shield ProtoShield Multi-functional Expansion Board Sensor Shield Module witn Four Digital Display for Arduino

Lm35

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 3B

C code

/*

/*

* Design_Assignment_3B.c

*

* Created: 10/16/2019 12:05:36 PM

* Author : Moriah Wingrove

*/

//TASK ONE: Write a C AVR program that will monitor the LM34/35 connected to an Analog pin (PC5) to display

//			the temperature in F on the serial terminal every 1 sec. Use a timer with interrupt delay for the

//			1 sec delay. Use a FTDI chip for serial to USB conversion.

//TASK TWO: Use the ATMEL Studio Data Visualizer or any charting program to display the values

#define F_CPU 16000000UL //16 Mhz.

#define BAUDRATE 9600 //Set Baud rate 9600.

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

#include <avr/io.h>

#include <avr/interrupt.h>

#include <stdio.h>

#include <stdlib.h>

#include <util/delay.h>

#include <math.h>

void read_adc(void); // Function Declarations

void adc_init(void);

void USART_init(void);

unsigned char USART_receive(void);

void USART_transmit(unsigned char data); //function to send through USART

void USART_tx_string( char *data );

volatile unsigned int adc_temp;

char outs[20];

uint8_t OVF_COUNT = 0; //initialize the overflow count for interrupt

uint8_t OVF_LIMIT = 250; //set the limit the count can reach to set 1 sec delay

int main(void) {

adc_init(); // Initialize the ADC (Analog / Digital Converter)

USART_init(); // Initialize the USART (RS232 interface)

USART_tx_string("Connected!\r\n"); // we're alive!

_delay_ms(125); // wait a bit

while(1)

{

read_adc();   // snprintf(outs,sizeof(outs),

//"%3d\r\n", adc_temp);

// print it

USART_tx_string(outs);

_delay_ms(1000); // wait a bit

}

}

void adc_init(void)

{

/** Setup and enable ADC **/

ADMUX = (0<<REFS1)| // Reference Selection Bits

(1<<REFS0)| // AVcc - external cap at AREF

(0<<ADLAR)| // ADC Left Adjust Result

(1<<MUX2)| // ANalog Channel Selection Bits

(0<<MUX1)| // ADC2 (PC2 PIN25)

(0<<MUX0);

ADCSRA = (1<<ADEN)| // ADC ENable

(0<<ADSC)| // ADC Start Conversion

(0<<ADATE)| // ADC Auto Trigger Enable

(0<<ADIF)| // ADC Interrupt Flag

(0<<ADIE)| // ADC Interrupt Enable

(1<<ADPS2)| // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);  // Timer/Counter1 Interrupt Mask Register

TIMSK1 |= (1<<TOIE1); // enable overflow interrupt

TCCR1B |= (1<<CS11)|(1<<CS10);  // native clock

TCNT1 = 49911;

}

/* READ ADC PINS */

void read_adc(void)

{

unsigned char i =4;

adc_temp = 0;

while (i--) {

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_temp+= ADC;

_delay_ms(50);

}

adc_temp = adc_temp /3; // Average a few samples

//int tempf= (adc_temp*5*100);

snprintf(outs,sizeof(outs),"%3d\r\n", adc_temp);// print it

USART_tx_string(outs);

}

void USART_init(void)

{

UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8);

UBRR0L = (uint8_t)(BAUD_PRESCALLER);

UCSR0B = (1 << RXEN0) | (1 << TXEN0);

UCSR0C = (3 << UCSZ00);

}

/* SEND A STRING TO THE RS-232 */

unsigned char USART_receive(void)

{

while(!(UCSR0A & (1 << RXC0)));

return UDR0;

}

void USART_transmit(unsigned char data)

{

while(!(UCSR0A & (1 << UDRE0)));

UDR0 = data;

}

void USART_tx_string(char* data)

{

while(*data != 0x00)

{

USART_transmit(*data);

data++;

}

}

ISR(TIMER1_OVF_vect)

{

OVF_COUNT++; //increment the overflow counter

if (OVF_COUNT == OVF_LIMIT) //check to see if the limit was reached

{

ADCSRA|=(1<<ADSC);	//start conversion

while((ADCSRA&(1<<ADIF))==0);//wait for conversion to finish

ADCSRA |= (1<<ADIF);

int a = ADCL;

a = a | (ADCH<<8);

a = (a/1024.0) * 5000/10;

a = (a*2)+32; //equation to convert celsius to farenheit. can't use used 2 instead of 9/5

a = a % 100;

USART_transmit((a/10)+'0');

a = a % 10;

USART_transmit((a)+'0');

USART_transmit('\n');

OVF_COUNT = 0; //reset overflow counter

}

TCNT0 = 16; //reset TOP

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

https://github.com/windew/Tiny_Dragons

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT