CPE301 – SPRING 2019

# Design Assignment 4A

Student Name: Moriah Wingrove

Student #: 5000156247

Student Email: Wingrove@unlv.nevada.edu

Primary Github address: https://github.com/windew/Tiny_Dragons.git

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega 328pb

pushbutton

HiLetgo Multi-nction Shield ProtoShield Multi-functional Expansion Board Sensor Shield Module witn

DC Toy Motor

INITIAL/MODIFIED/DEVELOPED CODE OF TASK 3B

C code

/*

* Design_Assignment_4A.c

*

* Created: 10/29/2019 7:55:13 PM

* Author : Moriah Wingrove

*/

//Task One: Write an AVR C program to control the speed of the DC Motor using a potentiometer connected to PC0.

//			Use an interrupt on a button (PC1/2/3) to stop and start the motor at each click. The minimum speed

//			of the motor should be 0 when pot is at minimum position and at maximum position should be 95%

//			of PWM value

#define F_CPU 8000000UL // clock is 8MHz

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

void init_adc(void);

int control; // controlling the motor with potentiometer

int toggle = 0; // to toggle the motor with the switch

int main()

{

DDRC = 0xFB; // set Port C as outputs, leave INT1(PC3) as interrupt

PORTC = 0x00; // pull-up pins

EIMSK = 0x02; // enable INT1

EIFR = 0x02; // enable interrupt flag

EICRA = 0X0C; // set interrupt on rising edge

sei(); // enable interrupts

init_adc(); // initiate adc

TCCR0A=0x83; // set fast PWM // clear OCR0A on MATCH

TCCR0B=0x03; // set prescaler to 1024

while (1)

{

while((ADCSRA&(1<<ADIF))==0); // wait for conversion

control = ADC*80/100; // ADC Conversion

OCR0A = control; // Output to converted value to 0CR0A

if(toggle == 0) // when switch is NOT pressed

{

PORTC = 0x00; //

}

}

}

void init_adc(void) // Initiate ADC function

{

ADMUX = (1<<REFS0); // Reference voltage at Aref

ADCSRA = (1<<ADEN)|(1<<ADSC)|(1<<ADATE)|(1<<ADPS2)|(1<<ADPS1)|(1<<ADPS0);

// from ADCRSA we Enable ADC, Start Conversion, Set prescalar as 128

}

ISR(INT1_vect)

{

toggle ^= 1; // toggle switch on INT1 Interrupt

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

https://github.com/windew/Tiny_Dragons

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT