CPE301 – SPRING 2019

# Design Assignment 4B

Student Name: Moriah Wingrove

Student #: 5000156247

Student Email: Wingrove@unlv.nevada.edu

Primary Github address: https://github.com/windew/Tiny_Dragons.git

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega 328pb

pushbutton

HiLetgo Multi-nction Shield ProtoShield Multi-functional Expansion Board Sensor Shield Module witn

stepper Motor

servo

INITIAL/MODIFIED/DEVELOPED CODE OF TASK 4B/1

C code

/*

* design_assignment_4B.c

*

* Created: 11/11/2019 10:21:04 AM

* Author : Moriah Wingrove

*/

//Task one: Write an AVR C program to control the speed of the stepper motor using a potentiometer connected to PC0. Use a timer in CTC

//			mode to control the delay

#define F_CPU 1000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

void init_adc(void); // ADC functions

void timer0_init(void); // ADC function for Timer 0 CTC

int control = 0; // converted ADC value to control motor speed

int main(void)

{

DDRB = 0xFF; // Set Port B for outputs

int wait = 0; // Wait is the delay

init_adc(); // Initialize ADC conversions

timer0_init(); // Initialize timer0 CTC function

while(1)

{

while((ADCSRA&(1<<ADIF))==0);

control = ADC*80/100; // ADC Conversion

// Stepper Motor function in half stepper mode (7 commands vs 4)

if (control < 1) // when control < 1 potentiometer is lowest voltage

{

wait = 0; // wait is the delay that controls the

PORTB = 0x0C;

_delay_ms (wait);

PORTB = 0x04;

_delay_ms (wait);

PORTB = 0x06;

_delay_ms (wait);

PORTB = 0x02;

_delay_ms (wait);

PORTB = 0x01;

_delay_ms (wait);

PORTB = 0x09;

_delay_ms (wait);

PORTB = 0x08;

_delay_ms (wait);

}

else if (control < 3)

{

wait = 100;

PORTB = 0x0C;

_delay_ms (wait);

PORTB = 0x04;

_delay_ms (wait);

PORTB = 0x06;

_delay_ms (wait);

PORTB = 0x02;

_delay_ms (wait);

PORTB = 0x01;

_delay_ms (wait);

PORTB = 0x09;

_delay_ms (wait);

PORTB = 0x08;

_delay_ms (wait);

}

else if (control < 4)

{

wait = 50;

PORTB = 0x0C;

_delay_ms (wait);

PORTB = 0x04;

_delay_ms (wait);

PORTB = 0x06;

_delay_ms (wait);

PORTB = 0x02;

_delay_ms (wait);

PORTB = 0x01;

_delay_ms (wait);

PORTB = 0x09;

_delay_ms (wait);

PORTB = 0x08;

_delay_ms (wait);

}

else if (control > 4) // when control > 4 potentiometer at max value

{

wait = 10;

PORTB = 0x0C;

_delay_ms (wait);

PORTB = 0x04;

_delay_ms (wait);

PORTB = 0x06;

_delay_ms (wait);

PORTB = 0x02;

_delay_ms (wait);

PORTB = 0x01;

_delay_ms (wait);

PORTB = 0x09;

_delay_ms (wait);

PORTB = 0x08;

_delay_ms (wait);

}

}

}

void init_adc(void) // Initiate ADC function

{

ADMUX = (1<<REFS0); // Reference voltage at Aref

ADCSRA = (1<<ADEN)|(1<<ADSC)|(1<<ADATE)|(1<<ADPS2)|(1<<ADPS1)|(1<<ADPS0);

// from ADCRSA we Enable ADC, Start Conversion, Set prescalar as 128

}

//  Using a timer in CTC mode to control the delay

void timer0_init()

{

TCCR0B |= (1 << WGM12)|(1 << CS11)|(1 << CS10); // Prescalar 64 set CTC mode

TCCR0A |= (1 << COM1A0); // Set timer in OCOA Pin Toggle Mode

TCNT0 = 0; // Initialize Counter

OCR0A = control; // OCROA reading ADC converted value

}

INITIAL/MODIFIED/DEVELOPED CODE OF TASK 4B/2

/*

* Design_Assignment_4B_task2.c

*

* Created: 11/13/2019 9:46:51 AM

* Author : Moriah Wingrove

*/

//Task two: Write an AVR C program to control the position of the Servo Motor using a potentiometer

//			connected to PC0. When pot value is 0 the servo is at position

//			0 deg. and when pot value is max (approx. 5V) the servo is at position 180 deg

#define F_CPU 16000000UL								/* Define CPU Frequency e.g. here its 8MHz */

#include <avr/io.h>									/* Include AVR std. library file */

#include <util/delay.h>								/* Include Delay header file */

void main()

{

//Configure TIMER1

TCCR1A|=(1<<COM1A1)|(1<<COM1B1)|(1<<WGM11);        //NON Inverted PWM

TCCR1B|=(1<<WGM13)|(1<<WGM12)|(1<<CS11)|(1<<CS10); //PRESCALER=64 MODE 14(FAST PWM)

ICR1=4999;  //fPWM=50Hz (Period = 20ms Standard).

DDRB|=(1<<PB1);   //PWM Pins as Out

while(1)

{

OCR1A=97;   //0 degree

_delay_ms(200);

OCR1A=316;  //90 degree

_delay_ms(200);

OCR1A=425;  //135 degree

_delay_ms(200);

OCR1A=535;  //180 degree

_delay_ms(200);

}

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Figure 1:Stepper board setup

Figure 2: Servo setup

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

https://github.com/windew/Tiny_Dragons

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT