CPE301 – FALL 2019

# Midterm 1

Student Name: Moriah Wingrove

Student #: 5000156247

Student Email: Wingrove@unlv.nevada.edu

Primary Github address: https://github.com/windew/Tiny_Dragons/

Directory: midterm

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB, FTDI, ESP01, LM35

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

- #define F_CPU 16000000UL //16 Mhz.

- #define BAUDRATE 115200 //Set Baud rate 9600.

- #define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 8UL))) - 1)

- #include <avr/io.h>

- #include <avr/interrupt.h>

- #include <stdio.h>

- #include <stdlib.h>

- #include <util/delay.h>

- #include <math.h>

- void read_adc(void); // Function Declarations

- void adc_init(void); //function that initializes ADC

- void USART_init(void); //initializes USART

- unsigned char USART_receive(void); //fn that receives USART

- void USART_transmit(unsigned char data); //function to send through USART

- void USART_tx_string( char *data );

- volatile unsigned int adc_temp;

- char outs[20];

- uint8_t OVF_COUNT = 0; //initialize the overflow count for interrupt

- uint8_t OVF_LIMIT = 250; //set the limit the count can reach to set 1 sec delay

- int main(void) {

- adc_init(); // Initialize the ADC (Analog / Digital Converter)

- USART_init(); // Initialize the USART (RS232 interface)

- USART_tx_string("AT\r\n"); //

- _delay_ms(500); // wait a bit

- USART_tx_string("AT+CWMODE=3\r\n");

- _delay_ms(500); // wait a bit

- USART_tx_string("AT+CWJAP=\"WIFI_NAME_HERE\",\"WIFI_PASSWORD_HERE\""); // // Log in WiFi

- _delay_ms(500);

- USART_tx_string("AT+CIPMUX=0\r\n");

- _delay_ms(500);

- /////////////////////////////////////////////////

- TCCR0A = 0x00; //normal operation

- TCCR0B |= (1 << CS02); //set prescalar to 256

- TCNT0 = 16; //TOP = 256-240 = 16

- TIMSK0 |= (1 << TOIE0);

- sei(); //enable interrupt

- ////////////////////////////////////////////////////////

- while(1)

- {

- //keep waiting

- }

- }

- void adc_init(void)

- {

- /** Setup and enable ADC **/

- ADMUX = (0<<REFS1)| // Reference Selection Bits

- (1<<REFS0)| // AVcc - external cap at AREF

- (0<<ADLAR)| // ADC Left Adjust Result

- (1<<MUX2)| // ANalog Channel Selection Bits

- (0<<MUX1)| // ADC2 (PC2 PIN25)

- (0<<MUX0);

- ADCSRA = (1<<ADEN)| // ADC ENable

- (0<<ADSC)| // ADC Start Conversion

- (0<<ADATE)| // ADC Auto Trigger Enable

- (0<<ADIF)| // ADC Interrupt Flag

- (0<<ADIE)| // ADC Interrupt Enable

- (1<<ADPS2)| // ADC Prescaler Select Bits

- (0<<ADPS1)|

- (1<<ADPS0);  // Timer/Counter1 Interrupt Mask Register

- }

- /* READ ADC PINS */

- void read_adc(void)

- {

- unsigned char i =4;

- adc_temp = 0;

- while (i--) {

- ADCSRA |= (1<<ADSC);

- while(ADCSRA & (1<<ADSC));

- adc_temp+= ADC;

- _delay_ms(50);

- }

- adc_temp = adc_temp /3; // Average a few samples

- //int tempf= (adc_temp*5*100);

- snprintf(outs,sizeof(outs),"%3d\r\n", adc_temp);// print it

- USART_tx_string(outs);

- }

- void USART_init(void)

- {

- UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8);

- UBRR0L = (uint8_t)(BAUD_PRESCALLER);

- UCSR0A = (1 << U2X0);   // double mode for more accuracy

- UCSR0B = (1 << RXEN0) | (1 << TXEN0);

- UCSR0C = (3 << UCSZ00);

- }

- /* SEND A STRING TO THE RS-232 */

- unsigned char USART_receive(void)

- {

- while(!(UCSR0A & (1 << RXC0)));

- return UDR0;

- }

- void USART_transmit(unsigned char data) {			// Function to transmit ASCII value into UDR0

- while (!(UCSR0A & (1 << UDRE0)));			// Keep Checking until  UDRE0 data register 'High' to break loop

- UDR0 = data;								// Store unsigned char serial data into UDR0

- }

- void USART_tx_string(char* data)

- {

- while(*data != 0x00)

- {

- USART_transmit(*data);

- data++;

- }

- }

- ISR(TIMER1_OVF_vect)

- {

- OVF_COUNT++; //increment the overflow counter

- if (OVF_COUNT == OVF_LIMIT) //check to see if the limit was reached

- {

- ADCSRA|=(1<<ADSC);	//start conversion

- while((ADCSRA&(1<<ADIF))==0);//wait for conversion to finish

- ADCSRA |= (1<<ADIF);

- int a = ADCL;

- a = a | (ADCH<<8);

- a = (a/1024.0) * 5000/10;

- a = (a*2)+32; //equation to convert celsius to farenheit. can't use used 2 instead of 9/5

- USART_tx_string("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n");	// Connect API KEy

- _delay_ms(500);

- USART_tx_string("AT+CIPSEND=51\r\n");

- _delay_ms(500);

- USART_tx_string("GET /update?key=QTJ31HQI0KR0Q5AP&field1=");// Send Value

- _delay_ms(500);

- a = a % 100;

- USART_transmit((a/10)+'0');

- a = a % 10;

- USART_transmit((a)+'0');

- _delay_ms(500);

- USART_transmit('\n');

- _delay_ms(500);

- OVF_COUNT = 0; //reset overflow counter

- }

- TCNT0 = 16; //reset TOP

- }

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.