CPE301 –Fall 2019

# Design Assignment Final

Student Name: Moriah Wingrove

Student #: 5000156247

Student Email: Wingrove@unlv.nevada.edu

Primary Github address: https://github.com/windew/Tiny_Dragons.git

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega 328pb

APDS9660

ESP-01

INITIAL/MODIFIED/DEVELOPED CODE OF TASK 6

C code

/*

* mid2.c

*

* Created: 12/12/2019 2:38:41 PM

* Author : Moriah Wingrove

*/

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

// Global constants for uart

#define BAUD 115200

#define FOSC 16000000

#define UBRR FOSC/8/BAUD-1

#define APDS9960_WRITE 0x72

#define APDS9960_READ 0x73

//include standard libraries

#include <avr/io.h>

#include <stdlib.h>

#include <stdio.h>

#include <util/delay.h>

#include <math.h>

//include custom libraries

#include "APDS9960_def.h"

#include "i2c_master.h"

//Function declarations

void getValues(void);

// void TIMER1_init();

void init_APDS9960(void);

void usart_init();

void USART_putstring(volatile unsigned char *StringPtr);

//AT commands

volatile unsigned char AT[] = "AT\r\n"; // Test

volatile unsigned char CWMODE[] = "AT+CWMODE=3\r\n"; // Set Wi-Fi mode

volatile unsigned char CWJAP[] = "AT+CWJAP=\"ZTE7\",\"hello702\"\r\n"; // Get Wi-Fi info

volatile unsigned char CIPSTART[] = "AT+CIPSTART=\"TCP\",\"184.106.153.149\",80\r\n"; // Establish connection with ThingSpeak

volatile unsigned char CIPSEND[] = "AT+CIPSEND=104\r\n"; // Set send function to 104

volatile unsigned char CIPMUX[] = "AT+CIPMUX=0\r\n"; // Enable connection

volatile unsigned char SEND_DATA[] = "GET /update?key=D4LE8GA6LD4PGK13&field1="; // Get Write Key

volatile unsigned char RESET[] = "AT+RST\r\n"; // Get AT Firmware info

volatile unsigned char LINEBREAK[] = "\r\n"; // end of temperature transmission

volatile unsigned char CLOSE[] = "AT+CIPCLOSE\r\n";

//string for colors

volatile unsigned char RedStr[10];

volatile unsigned char GreenStr[10];

volatile unsigned char BlueStr[10];

uint16_t redVal, greenVal, blueVal;

int main(void){

i2c_init();

usart_init(115200);

init_APDS9960();

//Start up Esp

//Start AT communication

_delay_ms(10);

USART_putstring(AT);					//send AT to the USART

//connect to network

_delay_ms(10);

USART_putstring(RESET);		//reset ESP

_delay_ms(10);

USART_putstring(AT);			//confirm communication

_delay_ms(10);

USART_putstring(CWMODE);		//WiFi mode = 3

_delay_ms(10);

USART_putstring(CWJAP);		//Send wifi login

while(1){

//getValues();

_delay_ms(10);

USART_putstring(CIPMUX);		//Single connection point

_delay_ms(10);

USART_putstring(CIPSTART);	// Connect to ThingSpeak

_delay_ms(10);

USART_putstring(CIPSEND);	// Declare send length 50

_delay_ms(10);

getValues();

USART_putstring(SEND_DATA);	// Connect to proper key

USART_putstring(RedStr);		// Send adc data

USART_putstring("&field2=");

USART_putstring(GreenStr);		// Send adc data

USART_putstring("&field3=");

USART_putstring(BlueStr);		// Send adc data

_delay_ms(1000);

}

return 0;

}

void init_APDS9960(void){

uint8_t setup;

i2c_readReg(APDS9960_WRITE, APDS9960_ID, &setup,1);

if(setup != APDS9960_ID_1) while(1);

setup = 1 << 1 | 1<<0 | 1<<3 | 1<<4;

i2c_writeReg(APDS9960_WRITE, APDS9960_ENABLE, &setup, 1);

setup = DEFAULT_ATIME;

i2c_writeReg(APDS9960_WRITE, APDS9960_ATIME, &setup, 1);

setup = DEFAULT_WTIME;

i2c_writeReg(APDS9960_WRITE, APDS9960_WTIME, &setup, 1);

setup = DEFAULT_PROX_PPULSE;

i2c_writeReg(APDS9960_WRITE, APDS9960_PPULSE, &setup, 1);

setup = DEFAULT_POFFSET_UR;

i2c_writeReg(APDS9960_WRITE, APDS9960_POFFSET_UR, &setup, 1);

setup = DEFAULT_POFFSET_DL;

i2c_writeReg(APDS9960_WRITE, APDS9960_POFFSET_DL, &setup, 1);

setup = DEFAULT_CONFIG1;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG1, &setup, 1);

setup = DEFAULT_PERS;

i2c_writeReg(APDS9960_WRITE, APDS9960_PERS, &setup, 1);

setup = DEFAULT_CONFIG2;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG2, &setup, 1);

setup = DEFAULT_CONFIG3;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG3, &setup, 1);

}

void getValues(void){

uint8_t redVH, redVL;

uint8_t greenVH, greenVL;

uint8_t blueVH, blueVL;

unsigned char i;

char dummy[10];

// Read red value

i2c_readReg(APDS9960_WRITE, APDS9960_RDATAH, &redVH, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_RDATAL, &redVL, 1);

// Read green value

i2c_readReg(APDS9960_WRITE, APDS9960_GDATAH, &greenVH, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_GDATAL, &greenVL, 1);

// Read blue value

i2c_readReg(APDS9960_WRITE, APDS9960_BDATAH, &blueVH, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_BDATAL, &blueVL, 1);

redVal = (redVH << 8) | redVL;

greenVal = (greenVH << 8) | greenVL;

blueVal = (blueVH << 8) | blueVL;

// Set max threshold values

if (redVal > 255){

redVal = 255;

}

if (greenVal > 255){

greenVal = 255;

}

if (blueVal > 255){

blueVal = 255;

}

itoa(redVal, dummy, 10); //convert char to ascii

for(i = 0 ; i < 10 ; i++){

RedStr[i] = dummy[i]; //move converted ascii

}

itoa(greenVal, dummy, 10); //convert char to ascii

for(i = 0 ; i < 10 ; i++){

GreenStr[i] = dummy[i]; //move converted ascii

}

itoa(blueVal, dummy, 10); //convert char to ascii

for(i = 0 ; i < 10 ; i++){

BlueStr[i] = dummy[i]; //move converted ascii

}

}

void usart_init() {

UBRR0H = ((UBRR) >> 8);

UBRR0L = UBRR;

UCSR0A |= (1<< U2X0); // divisor baud = 8

UCSR0B |= (1 << TXEN0); // Enable transmission

UCSR0C |= (1 << UCSZ01) | (1 << UCSZ00); // 8 bits

}

void USART_putstring(volatile unsigned char *StringPtr)

{

while ((*StringPtr != '\0')){

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = *StringPtr;

StringPtr++;

}

- }SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

https://github.com/windew/Tiny_Dragons

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT